DIVISION 10 - SPECIALTIES


This specification section uses numbered level paragraph styles, which were not included in versions of Word prior to Word 97.  In the interests of clarity, all paragraph styles are formatted flush left.

Specification editor’s choice items are shown in [square brackets]. (Optional) paragraphs denote items available at additional cost.

Use TAB to go DOWN one paragraph number level; SHIFT+TAB to go one paragraph number level UP.

SECTION 10677/105600 – STORAGE ASSEMBLIES – (SPACESAVER “STOREFRONTtm” SORTER UNITS)

PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

1.2 SUMMARY

A. This Section includes the following:

1. Sorter Unit Assemblies (Panel Assemblies for Front-End [“StoreFrontTM”] Shelving Units).

B. Related Work, Not Furnished:

1. Spacesaver( 4-post/case-type shelving units.

2. Spacesaver( manual, mechanical assist and motorized mobile storage units.

C. Related Sections:

1. [Section 105613 – Metal Storage Shelving]

2. [Section 105626 – Mobile Storage Shelving.]

D. Allowances:

E. Alternates:

1.3 REFERENCES    

A. American National Standards Institute (ANSI) Standards:

1. Applicable standards for fasteners used for assembly.

B. American Society for Testing and Materials (ASTM) Standards:

1. Applicable standards for steel sheet materials used for fabrication. 

C. American Institute Of Steel Construction (AISC) Standards:

1. Applicable standards for steel materials used for fabrication. 

1.4 DESCRIPTION

A. General: Accessory items consist of components designed and manufactured to fit Spacesaver( 4-post mobile shelving systems. Accessory items can be installed concurrently with installation of shelving systems or can be retrofitted in existing systems.

B. Sorter Unit Assemblies (for manual systems): Assemblies consist of self-edged, laminate panel, panel brackets, fastener, faced-end and clothespins.  Laminate panel, panel brackets and fasteners are designed to support the mobile system operator (manual handle).  Faced-end caps off the open end of the high-density mobile carriage to prevent debris and feet from getting caught inside the mobile carriage and to provide a more finished aesthetic. Clothespins retain the top of the front-end shelf section keeping it flush with the adjacent shelving range. 

C. Sorter Unit Assemblies (for mechanical assist systems): Assemblies consist of self-edged, laminate panel, stub shaft, variable shaft, coupler, panel brackets, fastener, anti-roll plate assembly and faced-end.  Laminate panel (routed for hand crank), panel brackets and fasteners are designed to support mobile system operator (hand crank).  Stub shaft, variable shaft and coupler together transfer the drive force from the mobile system hand crank to the mobile carriage when the hand crank is turned. Anti-roll plate assembly provides a series of detent locations (holes) which engage the safety lock (when the safety lock is activated) to prevent mobile system movement. Faced-end caps off the open end of the high-density mobile carriage to prevent debris and feet from getting caught inside the mobile carriage and to provide a more finished aesthetic.

D. Sorter Unit Assemblies (for motorized systems): Assemblies consist of [powered control head mounting brackets, fasteners, faced-end and clothespins.  Control head mounting brackets and fasteners are designed to support mobile system operator (Eclipse Powered SystemTM control head)] [laminate panel, panel brackets, fasteners, faced-end and clothespins.  Laminate panel, panel brackets and fasteners are designed to support mobile system operator (Eclipse Powered SystemTM control head and Aisle Entry Sensors)].   .  The faced-end caps off the open end of the end of a high-density mobile carriage to prevent debris and feet from getting caught inside the mobile carriage and to provide a more finished aesthetic. Clothespins retain the top of the front-end shelf section keeping it flush with the adjacent shelving range.

E. Finishes: 

1. Fabricated Metal Components and Assemblies: Manufacturer’s standard powder coat paint finish.

2. Fabricated Laminate Components and Assemblies: Manufacturer’s standard low-pressure or high-pressure laminate finishes.  

Sizes can be described in paragraph below or in a SCHEDULE attached as the last page of the section.

F. Sizes (for manual and mechanical assist systems):

1. Available in nominal widths of [12] [15] [18] [24] [30] [36] inches ([304MM] [381MM] [457MM] [609MM] [762MM] [914MM]).

2. Available in nominal heights of [12.75] [18.75] inches ([323MM] [476MM]).

G. Sizes (for motorized systems):

1. [Control head mounting bracket size: 6 inches (152 MM) wide by 7.5 inches (190 MM) high to maximize storage capacity on sorter unit shelving.]

2. [Laminate panel size: 

a. Available in nominal widths of [12] [15] [18] [24] [30] [36] inches ([304MM] [381MM] [457MM] [609MM] [762MM] [914MM]).

b. Available in nominal height of 17.25 inches (438MM) to accommodate Aisle Entry Sensors.]

1.5 PERFORMANCE REQUIREMENTS

A. Design Requirements: Provide units manufactured in sizes and dimensions, which will fit Spacesaver( mobile storage units and Spacesaver 4-post shelving. 

1.6 SUBMITTALS

A. Product Data: Submit manufacturer's product literature and installation instructions for each type of accessory required.  Include data substantiating that products to be furnished comply with requirements of the contract documents.

B. Shop Drawings: Show fabrication, assembly, and installation details including descriptions of procedures and diagrams.  Show complete extent of accessory installation layout including quantities, locations and types of accessory units required. Include notations and descriptions of all installation items and components.

1. Show installation details at non-standard conditions, if any.

2. Provide layout, dimensions, and identification of each unit corresponding to sequence of installation and erection procedures.

3. Provide installation schedule and complete erection procedures to ensure proper installation.

C. Samples: Provide minimum 3 inch (76MM) square example of each color and texture on actual substrate for each component to remain exposed after installation.

D. Selection Samples: For initial selection of colors and textures, submit manufacturer's color charts consisting of actual product pieces, showing full range of colors and textures available.

E. Warranty:  Submit draft copy of proposed warranty for review by the [Architect] [Architect/Engineer] [Engineer] [Designer].

F. Maintenance Data: Provide in form suitable for inclusion in maintenance manuals for storage units. Data shall include operating and maintenance instructions, parts inventory listing, purchase source listing, emergency instructions, and similar information.

1. Submit manufacturer's instructions for proper maintenance materials and procedures.

2. Submit manufacturer's printed instructions for maintenance of installed work, including methods and frequency recommended for maintaining optimum condition under anticipated use conditions. Include precautions against using materials and methods, which may be detrimental to finishes and performance.

G. [Reference List: Provide a list of recently installed storage unit accessories to be visited by owner, architect, and contractor. Intent of list is to aid in verifying the suitability of manufacturer's products and comparison with materials and product specified in this section.]

H. Sustainability Commitments:

1. To ensure product manufacturer is taking proactive measures to minimize their environmental footprint, comply with current and relevant legal and regulatory requirements, and striving to continually achieve environmental objectives, furnish:

a. Certification attesting ISO 14001:2015 Environmental Management Systems registration.

1.7 QUALITY ASSURANCE

A. Manufacturer Qualifications: Engage an experienced manufacturer who is ISO 9001 certified for the design, production, installation and service of sorter unit assemblies, carriage mounted high-density mobile storage units and support rails.  Furnish registration attesting to an ISO 9001 certified quality system.

B. Installer Qualifications: Engage an experienced installer who is a manufacturer's authorized representative for the specified products for installing carriages and anchoring shelving units to carriages.

1. Minimum Qualifications: 1-year experience installing systems of comparable size and complexity to specified project requirements.

2. Guaranteed 24 hour minimum response time to service call.

1.8 DELIVERY, STORAGE AND HANDLING

A. Follow manufacturer’s instructions and recommendations for delivery, storage and handling requirements.

1.9 PROJECT CONDITIONS

A. Field Measurements: Verify quantities of storage units scheduled to receive accessories before fabrication. Indicate verified measurements on Shop Drawings.  Coordinate fabrication and delivery to ensure no delay in progress of the Work.

B. Established Dimensions: Where field measurements cannot be made without delaying the work, establish dimensions and proceed with fabricating storage units without field measurements.  Coordinate construction to ensure actual dimensions correspond to established dimensions.

Sequencing and Scheduling paragraph can be omitted unless project conditions dictate that and incremental installation sequence is warranted or necessary.

1.10 [SEQUENCING AND SCHEDULING

A. Sequence storage shelving unit accessories with [installation of storage units] [other work] to minimize possibility of damage and soiling during remainder of construction period.

B. Schedule installation of specified accessories after finishing operations, including painting have been completed. 

C. Provide components, which must be built in at a time, which causes no delays general progress of the Work.

D. Pre-installation Conference: Schedule and conduct conference on project site to review           methods and procedures for installing  storage units including, but not limited to,           the following:

1. Recommended attendees include:

a. Owner's Representative.

b. Prime Contractor or representative.

c. The [Architect] [Architect/Engineer] [Engineer/Architect] [Engineer] [Designer].

d. Manufacturer's representative.

e. Subcontractors or installers whose work may affect, or be affected by, the work of this section.]

1.11 Warranty

A. Provide a written warranty, executed by Contractor, Installer, and Manufacturer, agreeing to repair or replace units, which fail in materials or workmanship within the established warranty period.  This warranty shall be in addition to, and not a limitation of, other rights the Owner may have under General Conditions provisions of the Contract Documents.

B. Warrant all installed accessories against defects in materials and workmanship for a period of five years from date of acceptance by the Owner.

A separate maintenance agreement paragraph may not be required since accessory items have few parts requiring long-term or continuing maintenance.

1.12 [MAINTENANCE]

A. [Provide manufacturer’s extended maintenance agreement for [____] [years] [months], commencing on the day the standard maintenance warranty ends.]

PART 2 -  PRODUCTS

2.1 MANUFACTURERS

A. General: Products are accessory components designed to be used with high-density mobile storage systems and 4-post shelving system products manufactured by Spacesaver Corporation and distributed by Spacesaver Intermountain, LLC. 14761 South Future Way Suite 101, Draper UT 84020. Phone: 844-800-6014.
2.2 BASIC MATERIALS

A. General: Provide materials and quality of workmanship, which meets or exceeds established industry standards for products specified. Use furniture grade sheet metal, wood panels, plastic laminate and fasteners for component fabrication unless indicated otherwise.  Material thicknesses/gauges are manufacturer’s option unless indicated otherwise.

2.3 MANUFACTURED COMPONENTS

A. Face Panels:

1. Materials: Plastic laminate clad particleboard with plastic edging on vertical edges. 

2. Finishes: [Selected from manufacturer’s standard available colors and patterns.] [(Optional) Selected by the [Architect] [Architect/Engineer] [Engineer] [Designer].

B. Shelving:

1.  [Case Type Shelving:]

2. [Four Post Shelving:

a. Design: Wedge-lock type consisting of uprights, shelves, and shelf supports, designed to be assembled without fasteners or clips. Shelves shall not have any holes on exposed surfaces. Front and back flanges shall be flush with outside faces of posts.  Design shall permit individual shelf adjustment and/or removal anywhere along the entire height of uprights. All shelves shall be designed to be interchangeable with any other module of the same width.

b. Materials and Workmanship: Fabricate units from Class 1, cold-rolled steel sheet with all bends sharp and true and no exposed “knife” edges.

All units shall be free of burrs, sharp edges and projecting hardware with smooth, non-abrasive surfaces and edges.

After fabrication, shelving shall exhibit no dents, “oil canning”, buckling or other surface irregularities.

c. Uprights: Formed from steel sheet to a hollow “tee” shape for intermediate supports and formed angles for end supports. Uprights shall have keyhole slots on inner wall only. Provide with sheet steel panels full height and depth of end uprights. Provide intermediate “tee” uprights between adjacent units.

d. Shelves: Form from sheet steel with flanges on all sides and return hem on front and back flanges. Ends shall be formed to clear inside of upright offset panels. Shelves shall be independently adjustable on both faces of double entry units. Provide all shelves with slots for file dividers [divider rods] [bin dividers].

e. Canopy Tops: Same construction as shelf units.

f. Shelf Supports: Form from heavy gauge steel sheet with four solid steel shoulder rivets, two per ear, that interlock with inner wall of uprights.

g. Nominal Shelf Dimensions:  

Standard Lengths: 36 inches (914MM), with 30, 42, or 48 inch (762, 1067, or 1219MM) sections used to meet project requirements.

Shelf Edge Vertical Profile: 3/4 inch (19MM).

Vertical Adjustment Increment: 1-1/2 inches (38MM).

Width of Intermediate Uprights: 2 inches (51MM).

Clearance Between Uprights: Nominal shelf section length minus 2 inches (51MM).

Levelness Of Completed Shelf Units: Maximum 1/8 inch (3.2MM) between bottom shelf and canopy top, measured along the edge of any upright in any direction.  

Number of Vertical Shelf Spaces: As indicated on the drawings [_____].

Vertical Shelf-To-Shelf Spacing: As indicated on the drawings [_____].

h. Load Carrying Capabilities: Provide shelf units capable of supporting 40 pounds per lineal foot (18kg/305MM) with maximum deflection of L/140. Shelves shall exhibit no permanent deflection under fully loaded conditions.

i. Accessories: Provide [____] file dividers per shelf.

2.4 FABRICATION

A. General: Coordinate fabrication and delivery to ensure no delay in progress of the Work.

2.5 FINISHES

A. Colors:  [Selected from manufacturer’s standard available colors.] [Provide in custom colors as selected by [Architect] [Architect/Engineer] [Engineer.]

B. Paint Finish: Provide factory applied electrostatic powder coat paint.  Meet or exceed specifications of the American Library Association.

C. Low-Pressure Laminate Finish: [Selected from manufacturer’s standard available colors and patterns.] [Provide in custom colors as selected by [Architect] [Architect/Engineer] [Engineer.]

D. High-Pressure Laminate Finish: [Selected from manufacturer’s standard available colors and patterns.] [Provide in custom colors as selected by [Architect] [Architect/Engineer] [Engineer.]

Describe optional (additional cost) finish options below, or delete paragraph.

E. [(Optional)  (Describe finish materials)]

PART 3 -  EXECUTION

3.1 EXAMINATION

A. Examine shelving units scheduled to receive accessories [with Installer present] for compliance with requirements for installation tolerances and other conditions affecting performance of specified accessory items.

B. Verify that intended installation locations of sorter unit units will not interfere with or block established required exit paths or similar means of egress once units are installed.

C. Proceed with accessory installation only after unsatisfactory conditions have been corrected.

3.2 INSTALLATION

A. General: Follow manufacturer’s written instructions for installation of each type of accessory item specified.

3.3 FIELD QUALITY CONTROL

A. Verify accessory unit alignment and plumb after installation. Correct if required following manufacturer’s instructions.

B. Remove components that are chipped, scratched, or otherwise damaged and which do not match adjoining work.  Replace with new matching units, installed as specified and in manner to eliminate evidence of replacement.

3.4 ADJUSTING

A. Adjust all accessories to provide smoothly operating, visually acceptable installation.

3.5 CLEANING

A. Immediately upon completion of shelving installation, clean components and surfaces.  Remove surplus materials, rubbish and debris resulting from shelving installation upon completion of work and leave areas of installation in neat, clean condition.

3.6 DEMONSTRATION/TRAINING

A. Schedule and conduct demonstration of installed accessory items and features with Owner's personnel.

B. Schedule and conduct maintenance training with Owner's maintenance personnel. Training session should include lecture and demonstration of all maintenance and repair procedures that end user personnel would normally perform.

3.7 PROTECTION

A. Advise Owner of additional protection needed to ensure that system will be without damage or deterioration at time of substantial completion.

You could use pre-printed schedules and simply add them as last page.  Add paragraph  3.8 SCHEDULES and add subparagraph: “A. Equipment Schedules, See next page.” or similar wording.

END OF SECTION

Sorter Unit Specifications -10677/ 105600
4
(2007 Spacesaver Corp

